Practical no. 06

IP Security (IPsec) Configuration: Configure IPsec on network devices to provide secure communication and protect against unauthorized access and attacks.

#take 6 PC – pc0, pc1, pc2, pc3, pc4, pc5

#take 2 switches – 2960-24TT switch0 , 2960-24TT switch1

#take 3 routers – 1941 router0 , 1941 router1 , 1941 router2

#connect PCs to switches and switches to router using Copper Straight –Through wire

#connect Router to router with Copper Cross-over

# IP assigning

- Network 1

- Pc0 – 192.168.1.10 default gateway 192.168.1.1

- Pc2 – 192.168.1.20 default gateway 192.168.1.1

- Pc3 – 192.168.1.30 default gateway 192.168.1.1

- Router0 – g0/1 – 192.168.1.1

- Network 2

- Pc1 – 192.168.3.10 default gateway 192.168.3.1

- Pc5 – 192.168.3.20 default gateway 192.168.3.1

- Pc4 – 192.168.3.30 default gateway 192.168.3.1

- Router1 – g0/1 – 192.168.3.1

- Network 3

- Router0 – g0/0 – 209.165.100.1

- Router2 – g0/0 – 209.165.100.2

- Network 4

- Router2 – g0/1 – 209.165.200.2

- Router1 – g0/0 – 209.165.200.1

# click on router0 and then go to the CLI

#Give name to the router

en

conf t

hostname R1

#same for router1

en

conf t

hostname R2

#for router2

en

conf t

hostname ISP

#for R1

R1(config-if)#exit

R1(config)#exit

R1#show version

You can see the security is none

R1#conf t

R1(config)#ip route 0.0.0.0  0.0.0.0  209.165.100.2

#for R2

R2(config-if)#exit

R2(config)#exit

R2#show version

You can see the security is none

R2#conf t

R2(config)#ip route 0.0.0.0  0.0.0.0  209.165.200.2

#now we have to add security to both routers

For R2

R2#conf t

R2(config)#license boot module c1900 technology-package securityk9  > accept yes

R2(config)#exit

R2#copy run startup-config

R2#reload

For R1

R1#conf t

R1(config)#license boot module c1900 technology-package securityk9  > accept yes

R1(config)#exit

R1#copy run startup-config

R1#reload

Now go to R1

R1>en

R1#conf t

R1(config)# access-list 100 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

Now go to R2

R2>en

R2#conf t

R2(config)# access-list 100 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255

Now go to R1

R1>en

R1#conf t

R1(config)# crypto isakmp policy 10

R1(config-isakmp)# encryption aes 256

R1(config-isakmp)# authentication pre-share

R1(config-isakmp)#  group 5

Now go to R2

R2>en

R2#conf t

R2(config)# crypto isakmp policy 10

R2(config-isakmp)# encryption aes 256

R2(config-isakmp)# authentication pre-share

R2(config-isakmp)#  group 5

Now again go to R1

R1(config)# crypto isakmp key secretkey address 209.165.200.1

Now again go to R2

R2(config)# crypto isakmp key secretkey address 209.165.100.1

#IPsec transform-set

Go to R1

R1(config)# crypto ipsec transform-set R1->R2 esp-aes 256 esp-sha-hmac

Go to R2

R2(config)# crypto ipsec transform-set R2->R1 esp-aes 256 esp-sha-hmac

#Crypto mapping

Router R1

Run following command all at once :

crypto map IPSEC-MAP 10 ipsec-isakmp

set peer 209.165.200.1

set pfs group5

set security-association lifetime seconds 86400

set transform-set R1->R2

match address 100

Router R2

Run following command all at once :

crypto map IPSEC-MAP 10 ipsec-isakmp

set peer 209.165.100.1

set pfs group5

set security-association lifetime seconds 86400

set transform-set R2->R1

match address 100

in R2

R2(config)#int g0/0

R2(config-if)#crypto map IPSEC-MAP

in R1

R1(config)#int g0/0

R1(config-if)#crypto map IPSEC-MAP

#check ISP router

show ip route

Check whether it is working or not

Check with simulation by sending packet from one PC to another

#packet successfully transfer from one PC to another