PRACTICAL NO.08

Intrusion Detection System: Set up and configure an intrusion detection system (IDS) to monitor network traffic and detect potential security breaches or malicious activities.

#take two PCs , three routers , two switches and one server

Connect router to each other using serial DTE cable ,and another connection using automatic cable

#Assign IP to PC 1

IP  :  192.168.1.4

Default Gateway  :  192.168.1.1

#Assign IP to PC 2 
IP  :  192.168.4.4

Default Gateway  :  192.168.4.1

#Click on router 1 (left)   > Open CLI

Int gigabitEthernet 0/0

Ip address 192.168.1.1 255.255.255.0

No shut

Int sesrial0/1/0

Ip address 192.168.2.1 255.255.255.0

No shut

#Click on router 0 (top)   > Open CLI

Int serial0/1/0

Ip address 192.168.2.2 255.255.255.0

No shut

Exit

Int serial0/1/1

Ip address 192.168.3.1 255.255.255.0

No shut

#Click on router 2 (right)   > Open CLI

Int serial0/1/0

Ip address 192.168.3.2 255.255.255.0

No shut

Exit

Int gigabitEthernet 0/0

Ip address 192.168.4.1 255.255.255.0

No shut

#Click on Router 1 (Left)

Router rip

Netw 192.168.1.0

Netw 192.168.2.0

Exit (2 times))

Show Ip route

#Click on router 0 (top)

Router rip

Netw 192.168.2.0

Netw 192.168.3.0

Show ip route

#Click on Router R2 (right)

Router rip

Netw 192.168.3.0

Netw 192.168.4.0

Show ip route

#Open server

Ip config   >   assign IP to it

IP  :  192.168.1.50

Default gateway  :  192.168.1.1

#Make sure HTTP and HTTPs service on of server

#Ping pc 1 to pc 2 (pc 1 can communicate with pc 2)

#Ping from pc 2 to pc 1(pc 2 can communicate with pc 1)

#Apply security

#Open router 1 (left)

Show version

On config mode

License boot module c1900 technology-package securityk9

Exit

Copy run startup-config

Reload

#Open Router R1 (left)

Mkdir ipdir

Conf t

Ip ips config location ipdir

Ip ips name iosips

Ip ips signauture-category

Just write “ ? “ (if not understand anything )

Category all

Retired true

Exit

Category  ?

Category ios_ips

Retired false

Category ios_ips basic

exit

Interface gigabitEthernet 0/0

Ip ips iosips out

Exit

Logging host 192.168.1.50

Service timestamps log datetime msec

Ip ips signature-definition

Signature 2004 0

Just write “ ? “

status

Retired false

Enable true

exit

Engine

Event action deny-packet-inline

exit

Do show ip ips all

Ping from pc 1 to pc 2 – (pc1  can communicate with pc2)

Ping from pc 2 to pc 1 – (pc2  can not  communicate with pc1)

Check  server syslog